File: CODES.DOC

+++++++++++++++++++++++++++++++++++++++++++++++++++

+  The DERIVE for Windows Utility File CODES.MTH  +

+++++++++++++++++++++++++++++++++++++++++++++++++++

---------------------------------------------------------------------------

Contributed by: Peter Schofield , Trinity and All Saints University College

Leeds, UK.  Email:  p.schofield@tasc.ac.uk                        July 1998

---------------------------------------------------------------------------

This document file explains ways in which the DERIVE for Windows Utility file

CODES.MTH provides a set of powerful tools for generating and displaying some of the

large matrices and tables involved in an introduction to a study of error correcting

codes. Examples of how this can be carried out are given for: standard form generator

and parity-check matrices; codewords and minimum weights; syndrome look-up tables;

cyclic codes; Hamming codes; Reed-Muller codes. The file CODES.MTH operates best with

the powerful matrix display facilities of DERIVE for Windows.

1.	INTRODUCTION

Some basic understanding of error correcting codes will be assumed. This can be found

in any good introduction to the topic. For example:

Hill, R. A First Course in Coding Theory. Oxford University Press (reprinted 1997)

Baylis, J. Error Correcting Codes. A Mathematical Introduction. Chapman & Hall (1998)

Open University. MT365 Graphs, Networks and Designs (Design 3: Design of Codes). OU (1997)

2.	STANDARD FORM GENERATOR AND PARITY-CHECK MATRICES

2.1	Basics

For a given redundancy matrix A the standard form generator and parity-check matrices

for the corresponding linear code C are given by:

G =  [ IdimA  A]		H  =  [-A`  Idim.A`]

where -A` is the transpose of -A and "In" is the nxn identity matrix. Associated with

C is the set of codewords, the minimum weight of all non-zero codewords (which for a

linear code is equal to its minimum distance between codewords), and the syndrome

look-up table.

2.2	Utilities

The Utility file CODES.MTH provides the following tools:

q -  variable storing the base prime modulus for the code;

QU(a) - reduces all elements of the matrix a modulo q;

G(a) - constructs the generator matrix for the code with redundancy matrix a;

H(a) - constructs the parity-check matrix for the code with redundancy matrix a;

RAWDATA(g) - constructs the raw data (a set q-ary numbers) for the code with

generator matrix g

CODEWORDS(g) - constructs a matrix of all codewords for the code with generator matrix g;

MINWT(cg) - calculates the minimum weight of all of the non-zero codewords for the code

with matrix cg of codewords;

SLT1(h) - constructs a matrix of syndromes and coset leaders for all single errors for the

code with parity-check matrix h;

SLT2(h) - constructs a matrix of syndromes and coset leaders for all double errors for the

code with parity-check matrix h;

TABLE1(h) - constructs the full syndrome look-up table for up to 1 error for the code with

parity-check matrix h;

TABLE2(h) - constructs the full syndrome look-up table for up to 2 errors for the code with

parity-check matrix h.

2.3	Operating

Example 1. Generating the codewords from raw data for a binary code

Load the Utility file CODES.MTH and set the base modulus by   q:=2

Assign a redundancy matrix to a variable a by   a:=[[1,1,1,1,0],[1,1,1,0,1],[1,1,0,1,1]]

To constuct the standard form generator matrix simplify G(a)

To construct the raw data for this code simplyfy RAWDATA(G(a))

The codewords for the code are generated by taking products of the raw data and G(a).

G(a) is in standard form, and so the codewords consist of the raw data followed

by a set of check-digits determined by the redundancy (matrix) of the code.

To display this using CODES.MTH simplify [RAWDATA(G(a)),QU(RAWDATA(G(a)).G(a)]

or simplify [RAWDATA(G(a)),CODEWORDS(G(a))].

Example 2. Decoding using the ternary repetition code R6 (each ternary digit is sent six times)

Set the base modulus by   q := 3.

Assign a new redundancy matrix to the variable a by   a := [[ 1, 1, 1, 1, 1 ]]

To display the standard form generator and parity-check matrices for the code (R6) and to

check these are orthogonal simplify:   [ G(a), H(a), QU(G(a) . H(a)`) ].

By simplifying MINWT(CODEWORDS(G(a)) the full set of codewords plus the minimum weight for

the code can be displayed. First highlight CODEWORDS(G(a)) and simplify, then simplify the

result.

The minimum weight of R6 is 6 and so, using R6, it is possible to attempt to correct up to

2 errors (or detect up to 5 errors). The full syndrome look-up table for R6 (73 rows) can

be displayed by simplifying TABLE2(H(a)).

The principles of error correcting can also be illustrated with DERIVE assistance. For

ternary R6, unit vectors (matrices) for the code can be constructed using:

U(i):=[VECTOR(IF(j=i, 1, 0), j, 6)]. Note: these vectors have a 1 in the ith position and

0 for every other entry.

If a codeword is sent, for example c:=[[2, 2, 2, 2, 2, 2]] in ternary R6, then simplifying

QU(c.H(a)`) will result in a zero matrix. However, suppose c is sent but

c1 := [[0, 2, 1, 2, 2, 2]] is received. That is, there are errors of 1 and 2 (modulo 3) in

the first and third digits, respectively. The coset leader for the coset cotaining c1

is U(1)+2U(3), and so simplifying [QU((U(1)+2U(3)).H(a)`), QU(c1.H(a)`)] will show

that both  U(1)+2U(3) and c1 have the same syndrome. Therefore, they both belong to the

same coset. Hence, c1 will be corrected to the codeword  QU(c1-(U(1)+2U(3))) = c.

Other standard forms for codes can be studied by assigning new values to q and/or a.

3.	CYCLIC CODES

3.1 	Background

A length n cyclic code is a linear code such that if  a0a1...an-2an-1 is a codeword then

an-1a0a1...an-2 is also a codeword. Consequently, in a cyclic code, any cyclic shift of

a codeword is also a codeword. Cyclic codes are closely associated with principal ideals

in Fn[x], where  F is a finite field and Fn[x] is the set of all remainders

r(x)modulo(x^n - 1) ; deg(r(x)) < n. Such an ideal has a unique generator g(x)

(a monic polynomial of least degree) and g(x) will be a factor of (x^n - 1) over F.

Therefore, to find a generator polynomial for an ideal we first need to find a factor

of (x^n - 1). Using the tools already provided by DERIVE it is not difficult to test for

factors of polynomials over GF(prime). Once a generator polynomial g(x) for an ideal has

been found, a parity-check matrix for the corresponding cyclic code can be calculated

from the polynomial h(x) which satisfies: g(x).h(x) = (x^n - 1). The generator and

parity-check matrices for the corresponding cyclic code can now be found by a

straightforward algorithmic method.

3.2	Utilities

The DERIVE Utility file CODES.MTH provides the following:

q - key variable for storing the base prime modulus for the code;

MPOL(p) - constructs the reduced  polynomial modulo q from polynomial p;

FACTQN(p,pq) - tests whether or not the polynomial p is a factor of polynomial pq modulo q;

CG(p,n) - constructs the generator matrix for the length n cyclic code with polynomial

generator p;

CH(p,n) - constructs the parity-check matrix for the length n cyclic code with polynomial

generator p.

3.3	Operating

For example, to set up the file up to investigate a binary cyclic code of length 7 with a

generator polynomial x^3+x+1, assign 2 to q and  x^3+x+1  to p. Simplifying

FACTQN(p,x^7-1) will first identify whether or not p is a factor of (x^7 - 1). If p

is a factor, then FACTQN(p,x^7-1) will display [p, h(x)] , otherwise it displays

"not a factor". For this example, FACTQN(p,x^7-1) outputs: [x^3+x+1,x^4+x^2+x+1].

Simplifying [CG(p,7),CH(p,7),QU(CG(p,7).CH(p,7)`)] will output the generator CG(p,7) and

parity-check CH(p,7) matrices for the corresponding cyclic code (plus a check that they are

orthogonal). (It is also instructive to simplify [CG(p,7),CH(p,7),QU(CG(p,7).CH(p,7)`)]

for a polynomial p which is not a factor of (x^7 - 1)).

From this point on it is possible to use CODEWORDS(g) to generate the codewords

for the cyclic code. Also, see the DERIVE file CODES.DMO for a demonstration of how to

simulate a shift register for generating codewords of a cyclic code.

4.   	HAMMING and REED-MULLER CODES

4.1	Hamming Codes

A binary Hamming code is characterised by a parity-check matrix whose columns consist of

all of the distinct non-zero binary length r vectors.  Also,  HAM(r) has minimum weight

of 3, and so it can attempt to correct all single errors. To employ a straightforward

error checking technique it is usual to arrange the columns of  HAM(r) in numerical order.

In this case, HAM(r) will not be in standard form. In this form the error correcting

technique for binary HAM(2) is straightforward. If the syndrome of a received vector in

binary is equivalent to decimal i, then the error has occurred in the ith position of the

vector. SLTHAM(r) generates a full syndrome indexed look-up table for HAM(r) to highlight

this form of error correction.

Hamming codes are an important subclass of error correcting codes. They are codes

for which each received vector can be corrected to a unique (nearest neighbour) codeword.

A code with this property is called perfect.

(For binary Hamming codes assign q:=2.)

The DERIVE Utility file CODES.MTH provides:

HAM(r) - constructs a parity-check matrix for a Hamming code in rows;

(That is, HAM(r) is of dimension (2^r-1)xr . Therefore, transpose to display.)

SLTHAM(r) - constructs the full indexed syndrome look-up table for HAM(r).

Two more predicates:

SWAP(a,i,j) - swap rows i and j of matrix a;

DEL(a,i) - delete last i rows of matrix a,

have been included so that the operator can carry out a "do it yourself" construction

of a corresponding non-standard generator matrix for HAM(r). See CODES.DMO for an example

of how this can be done for HAM(4).

q-ary Hamming codes can also be defined for any prime modulus q. Their construction

for q>2 is not quite so straightforward as for q=2. To construct a parity-check

matrix (using CODES.MTH) for a ternary Hamming code assign q:=3, etc. The error

correcting technique for HAM(r) modulus q>2 can still be applied on the position

(and multiple) of the syndrome of the received vector. SLTHAM(r) will still be helpful

in these more general cases.

4.2	Reed-Muller Codes

One way of generating Reed-Muller codes is by means of the (u/u+v) construction.

Suppose  u = u1 ... un   and  v = v1 ... vn are two length n codewords then

(u/u+v) = u1 ... un u1+v1  ... un+vn.

Any linear code C can be classified as an [n, k, d] code where: n = length of codewords;

k = dimension; d = minimum distance. From two linear codes C1 [n ,k1,d1] and C2 [n ,k2,d2]

we can form the code (C1/C2) using the (u/u+v) construction as follows:

(C1/C2) = { (u/u+v) : u belongs to C1, v belong to C2 }.

Two basic results of this construction are:

(i)  (C1/C2) is a [2n, k1 + k2, min(2d1, d2)] code;

(ii)  if G1 and G2 are the generators of C1 and C2, respectively, then a generator matrix

for (C1/C2) is

G1 G1

O  G2      where "O" is the k2xn zero matrix.

Reed-Muller codes can be constructed using the two "simple codes" (as seeds):

RM(0,m) = the binary repetition code of order 2m ;   RM(m,m)= the full code of length 2m

together with the following double recursion using the  (u/u+v) construction:

RM(r, m) = (RM(r, m-1)/RM(r-1,m-1))      for r, m > 0    (#)

RM(0,m) is a [2m, 1, 2m] code with a generator matrix consisting of a single row of 2m units

and RM(m,m) is a [2m, 2m, 1] code with the 2mx2m identity matrix as generator. By combining

these codes with the (u/u+v) construction Reed-Muller codes possess a whole variety of lengths

of codewords, numbers of codewords and minimum distances.

(To operate QU(a) with Reed-Muller codes assign q:=2.)

The file CODES.MTH contains:

RM(r,m) - constructs the generator matrix for the (r,m) Reed-Muller code.

This is straightforward conversion of the initial matrices and double recursion (#) into a

DERIVE expression designed to construct generator matrices for Reed-Muller codes.

For example, simplifying RM(1, 5) constructs the matrix of a [32, 6, 16] code. That is, it

has 2^6 codewords of length 32 and it can detect up to 15 errors or attempt to correct up

to 7 errors.

Another basic result of  Reed-Muller codes is that RM(m-r-1,m) generates the dual

(orthogonal) code of RM(r,m). For example, to verify that RM(2,5) is self-dual,

simplify QU(RM(2,5).RM(2,5)`) to obtain a zero matrix.

This opens up a whole variety of coding and error correcting (or detecting) activities.

For example, by making use of combinations of unit vectors (matrices) of form

U(i) := [VECTOR(IF(i = j, 1, 0), j, 32)], RM(3, 5) can be used as the parity-check matrix

for correcting up to 7 errors (or detecting up to 15 errors) in RM(1,5).

See also CODES.DMO on how to generate Reed-Muller tables for interpreting the double

recursion (#).

================== END OF FILE =================